PROJECT REPORT

Assembly Language and Computer Architecture Lab – IT3280e

Group 2

Ngụy Quang Sơn – 20225998

Lò Đức tài – 20225999

- Problem 8: Pocket Calculator

- Problem description

Use a keypad and two 7-segment displays (Digital Lab Sim) to build a simple calculator supporting operations +, -, *, /, and % on integers.

Requirements:

- Use keypad keys for operations:

- a for addition (+)

- b for subtraction (–)

- c for multiplication (*)

- d for division (/)

- e for modulo (%)

- f for equals (=)

- When entering a number, display the last two digits of the number on the two 7-segment displays.

Example of entering 123: 1 → 01, 2 → 12, 3 → 23.

- After entering a number, select an operation (+, –, etc.) and press f to calculate.

- Allow consecutive calculations (like the Windows Calculator app)

- How to use

- Open Digital Lab Sim (connect to program) and run the program

- Click the number in the numpad and the corresponding number will appear on the

Seven-Segment Display LED (LED on the right side)

- Choose an operator (see the problem description to see what key is corresponding to what character. Example: a is addition, b is subtraction,…)

- Choose the number again for calculating. (Exception: If the user does not click any number key, the second number will be default to zero)

- Click f to compute the expression. The result will be displayed on the screen. (

*Notice: The next calculation will depend on previous calculation (the first number = result of previous calculation) so remember to enter next operator before enter the number (if not the operator is set default for addition calculation)

3. How the program work

Data section:

- Number: Array of 7-segment display codes for digits 0-9.

- Numpad: Array of key codes for a 4x4 keypad.

- Message: Various messages to be displayed.

Text section:

- init: initializes the registers and sets up the initial state of the calculator.

- main: Sets the initial display of two LEDs to 0

- Enter Number Loop:

- enterNumber: Scans the numpad on each row to detect which key is pressed.

- loop_enter: Iterates through the rows of the numpad.

- checkNumPad: Determines which key was pressed by comparing the input with the NumPad array (the number pressed is t2)

- process_key: Determines if a number or an operator key was pressed and jumps to the appropriate label.

- Arithmetic Operation:

- addition, subtraction, multiplication, division, modulo: Set the operator and change state

- equals: Checks which operator was set and jumps to the corresponding calculation.

- Calculation Functions:

- calc_add, calc_sub, calc_mul, calc_div, calc_mod: Perform the respective arithmetic operations, check for overflow, and display the result.

- Display Number part:

- Display_Number: Update two LEDs with the enter number.

- update_number1: Update the first number (s4)

- update_number2: Update the second number (s5)

- sleep: pause the program for 100ms

- reset_row: resets the row scanning to the first row.

- next_cal: Prepares for the next calculation by updating s4 with the result and resetting s5.

- error_handling: handle the case like divide, mod by 0 or overflow occur.

4. Source Code

.eqv IN_ADDRESS_HEXA_KEYBOARD 0xFFFF0012 # Receive row and column of the key pressed, 0 if not key pressed

.eqv OUT_ADDRESS_HEXA_KEYBOARD 0xFFFF0014

.eqv LEFT_7SEGMENT_LED 0xFFFF0011

.eqv RIGHT_7SEGMENT_LED 0xFFFF0010

.data

Number: .word 0x3F, 0x06, 0x5B, 0x4F, 0x66, 0x6D, 0x7D, 0x07, 0x7F, 0x6F

NumPad: .word 0x11, 0x21, 0x41, 0x81, 0x12, 0x22, 0x42, 0x82, 0x14, 0x24, 0x44, 0x84, 0x18, 0x28, 0x48, 0x88

NumPadEnd: .word

mess0: .asciz " and "

mess1: .asciz " is: "

addMess: .asciz "The addition of "

subMess: .asciz "The subtraction of "

mulMess: .asciz "The multiplication of "

divMess: .asciz "The division of "

modMess: .asciz "The modulo of "

newline: .asciz "\n"

prompt1: .asciz "\nContinue the calculation....\n"

prompt2: .asciz "\nCan not divide by 0. Reset the calculator.\n"

prompt3: .asciz "\nCan not mod 0. Reset the calculator.\n"

prompt4: .asciz "\nOverflow occurred. Reset the calculator.\n"

.text

init:

li s0, IN_ADDRESS_HEXA_KEYBOARD

li s1, OUT_ADDRESS_HEXA_KEYBOARD

li s2, LEFT_7SEGMENT_LED

li s3, RIGHT_7SEGMENT_LED

li s4, 0 #number1 : x1

li s5, 0 #number2 : x2

li s7, 0 #check state

li s8, 10 # operator: + = 10, - = 11, * = 12, / = 13, % = 14

main:

#store previous value

la a1, Number

lw s6, 0(a1) #start previous led = 0

sb s6, 0(s2) #default left Led display 0

sb s6, 0(s3) #default right Led display 0

enterNumber:

li t0, 0x01 #check row i (start from row i)

li t1, 0x10 #if = row 5 then loop again

loop_enter:

beq t0, t1, reset_row

sb t0, 0(s0) #reassign each row

lbu a1, 0(s1) #read enter button

beq a1, zero, update_loop_enter

la a2, NumPad # a2 = address(Numpad[0])

la a3, NumPadEnd

addi a3, a3, -4 # a3 = address(NumPad[n-1])

li t2, 0 #number clicked

checkNumPad: #function to find which number was clicked (determined by t2)

lw t3, 0(a2) #t3 = Numpad[i]

beq a1, t3, process_key #if button clicked is Numpad[i] then go find which button was clicked

addi a2, a2, 4

addi t2, t2, 1

j checkNumPad

process_key:

li t3, 10 #number range

blt t2, t3, Display_Number #if t2 < 10 then number was clicked

#else the operator is clicked

beq t2, t3, addition

addi t3, t3, 1

beq t2, t3, subtraction

addi t3, t3, 1

beq t2, t3, multiplication

addi t3, t3, 1

beq t2, t3, division

addi t3, t3, 1

beq t2, t3, modulo

addi t3, t3, 1

beq t2, t3, equals

update_loop_enter:

slli t0, t0, 1 #row i + 1

j loop_enter

addition:

li s8, 10 # operator +

li s7, 1 #change state

j main

subtraction:

li s8, 11 #operator -

li s7, 1 #change state

j main

multiplication:

li s8, 12 #operator x

li s7, 1 #change state

j main

division:

li s8, 13 #opearor /

li s7, 1 #change state

j main

modulo:

li s8, 14 #operator %

li s7, 1 #change state

j main

equals: #check operator

li t3, 10

beq s8, t3, calc_add

addi t3, t3, 1

beq s8, t3, calc_sub

addi t3, t3, 1

beq s8, t3, calc_mul

addi t3, t3, 1

beq s8, t3, calc_div

addi t3, t3, 1

beq s8, t3, calc_mod

calc_add:

add a0, s4, s5

blt a0, s4, error_overflow #check for overflow

blt a0, s5, error_overflow #check for overflow

li a7, 4

la a0, addMess

ecall

li a7, 1

add a0, zero, s4

ecall

li a7, 4

la a0, mess0

ecall

li a7, 1

add a0, zero, s5

ecall

li a7, 4

la a0, mess1

ecall

li a7, 1

add a0, s4, s5

ecall

j next_cal

calc_sub:

sub a0, s4, s5

bgt a0, s4, error_overflow #check for overflow

li a7, 4

la a0, subMess

ecall

li a7, 1

add a0, zero, s4

ecall

li a7, 4

la a0, mess0

ecall

li a7, 1

add a0, zero, s5

ecall

li a7, 4

la a0, mess1

ecall

li a7, 1

sub a0, s4, s5

ecall

j next_cal

calc_mul:

mul a0, s4, s5

div t3, a0, s5

bne t3, s4, error_overflow #check for overflow

li a7, 4

la a0, mulMess

ecall

li a7, 1

add a0, zero, s4

ecall

li a7, 4

la a0, mess0

ecall

li a7, 1

add a0, zero, s5

ecall

li a7, 4

la a0, mess1

ecall

li a7, 1

mul a0, s4, s5

ecall

j next_cal

calc_div:

beq s5, zero, error_division

li a7, 4

la a0, divMess

ecall

li a7, 1

add a0, zero, s4

ecall

li a7, 4

la a0, mess0

ecall

li a7, 1

add a0, zero, s5

ecall

li a7, 4

la a0, mess1

ecall

li a7, 1

div a0, s4, s5

ecall

j next_cal

calc_mod:

beq s5, zero, error_modulo

li a7, 4

la a0, modMess

ecall

li a7, 1

add a0, zero, s4

ecall

li a7, 4

la a0, mess0

ecall

li a7, 1

add a0, zero, s5

ecall

li a7, 4

la a0, mess1

ecall

li a7, 1

rem a0, s4, s5

ecall

j next_cal

Display_Number:

#print number entered on LED

la a1, Number #a1 = address of Number[0]

slli t3, t2, 2

add a1, a1, t3

lw a2, 0(a1)

sb a2, 0(s3) #Right Led display the value entered

sb s6, 0(s2) #Left Led display the previous value entered

mv s6, a2 #update previous value = value entered

#update value base on state

li t3, 10

beq s7, zero, update_number1 #update the first Number

j update_number2	#update the second Number

update_number1:

mul s4, s4, t3	#x1 *= 10

add s4, s4, t2 #x1 += x (value was entered)

j sleep

update_number2:

mul s5, s5, t3 #x2 *= 10

add s5, s5, t2 #x2 += x (value was entered)

j sleep

sleep:

li a0, 100 # sleep 100ms

li a7, 32

ecall

reset_row:

li t0, 0x01

j loop_enter

next_cal:

mv s4, a0 #update x1 = calculation result

li s5, 0 #reset x2 = 0

li a7, 4

la a0, prompt1

ecall

li s8, 10 #reset operator to default "+"

j main

#handle case divide or mod by 0

error_handling:

error_division:

li a7, 4

la a0, prompt2

ecall

j init

error_modulo:

li a7, 4

la a0, prompt3

ecall

j init

#handle overflow

error_overflow:

li a7, 4

la a0, prompt4

ecall

j init

- Code Demo

Start implement:

Press button: 1

Press button: 2

Press button: b (perfrom subtraction)

When performing operator, reset LED to 0 for entering next number

Press button: 5

Press button: f (equals)

After press, it will print out the value on the screen and continue the next calculation.